Checking the Adaptive Compression Controller on the PINE S7

Board: XC7S15-FTGB196-1.  Bitstream already generated; this sheet is only for checking that it works.

# 1. Load the bitstream

- Connect the board to the PC with the micro-USB cable. The power LED should light.

- Set all 16 slide switches to OFF (down).

- In Vivado: Open Hardware Manager > Open Target > Auto Connect > Program Device, choose pine_s7_top.bit, click Program.

- The DONE LED on the board turns green. The 7-segment display should show 0.000 (every block is empty).

# 2. Controls

| Control | What it does |
| --- | --- |
| SW2 SW1 SW0 | Selects the test pattern 0 to 7 (binary, SW0 is the lowest bit). |
| SW7 SW6 SW5 SW4 | Block address 0 to 15: the block you write to and the block you are viewing. |
| SW15 to SW12 | Selects which 16-bit slice (0 to 15) of the stored compressed word shows on the LEDs. |
| KEY_0 | Writes the selected pattern into the selected block. One press = one write. |
| KEY_3 | Reset. Clears the block table. |
| 7-segment | Shows mode.size, for example 1.068. Mode: 0 = RAW, 1 = DELTA4, 2 = DELTA8, 3 = RLE. Size is in bits (a raw block is 128). |

# 3. Test 1: all 8 patterns

Leave SW7 to SW4 and SW15 to SW12 all OFF (block 0, slice 0). For each row: set SW2 SW1 SW0, press KEY_0 once, then compare the display and LEDs. 1 means the switch is ON (up).

| Pattern | SW2 SW1 SW0 | Display | LED15 ... LED0 (1 = lit) | Pass |
| --- | --- | --- | --- | --- |
| 0 | 0 0 0 | 1.068 | 0001 0001 0000 0001 |  |
| 1 | 0 0 1 | 1.068 | 0001 0011 0110 0100 |  |
| 2 | 0 1 0 | 0.128 | 0111 1000 0110 0100 |  |
| 3 | 0 1 1 | 3.064 | 0000 1000 0100 0001 |  |
| 4 | 1 0 0 | 3.016 | 0001 0000 0000 0000 |  |
| 5 | 1 0 1 | 0.128 | 1111 1111 0000 0000 |  |
| 6 | 1 1 0 | 3.032 | 0000 1000 0000 0000 |  |
| 7 | 1 1 1 | 1.068 | 1111 1111 1111 0000 |  |

What this proves: the controller picks the right method for each data type. Smooth ramps use DELTA4 (68 bits), repeated values use RLE (16 to 64 bits), and irregular or alternating data stays RAW (128 bits). Pattern 4 (all zeros) is the best case, 8 times smaller.

# 4. Test 2: blocks are remembered

- Set SW7 to SW4 = 0001 (block 1), pattern 0 (SW2 SW1 SW0 = 000). Press KEY_0.

- Set SW7 to SW4 = 0010 (block 2), pattern 3 (011). Press KEY_0.

- Now only move SW7 to SW4. Block 1 should show 1.068 and block 2 should show 3.064.

- Check block 0 and block 3 as well. Block 3 was never written, so it should show 0.000.

# 5. Test 3: view the stored data

Write pattern 3 (011) to block 0, then change SW15 to SW12:

| SW15 to SW12 | LED15 ... LED0 |
| --- | --- |
| 0000 (slice 0) | 0000 1000 0100 0001 |
| 0001 (slice 1) | 0000 0011 0100 0010 |

These two slices are the stored run-length pairs for pattern 3 (value 41 repeated 8 times, 42 repeated 3 times, and so on).

# 6. Test 4: reset

Press KEY_3. Every block should now show 0.000. The LEDs may still show old data after a reset. That is expected, because only the block table is cleared; the display reading is what counts.

# 7. If something is wrong

| Symptom | What to try |
| --- | --- |
| Display blank, dim or garbled; or all segments lit | Display polarity is not in the board manual. In pine_s7_top.v change SEG_ACTIVE_LOW and/or SEL_ACTIVE_LOW from 1 to 0 (try each), then rebuild the bitstream. |
| Display works but digits are in the wrong order | Swap the SEL_DISP1 to SEL_DISP4 pin assignments in the XDC file, then rebuild. |
| Display never changes after KEY_0 | Hold the button for about half a second (it is debounced). Buttons read low when pressed. Confirm DONE LED is green. |
| Pattern results are off by a bit | Check that the switch is really in the position you expect. Try flipping the up/down convention. |
| Mode digit right but size wrong | Re-check which block address (SW7 to SW4) you are viewing versus the one you wrote. |
| Nothing at all, DONE LED off | Reprogram the device; make sure you selected pine_s7_top.bit. |

All expected values above come from the simulation of the same design. If Test 1 matches on the board, the compression logic is working on the FPGA.